Windsor Middle School Gymnasium Renovation RFP

The Town of Windsor is seeking proposals from qualified firms who are licensed and insured to conduct and oversee renovation work on the existing Windsor Middle School gymnasium.

# Scope of Work

- Repair and seal (point up) cracks in exterior masonry.  Note in particular the large cracks on the (south) side of the building next to the new school where the vertical brick buttress sections meet the other parts of the wall. Replace all exterior brick pilaster cap flashing.

- Replace 7 exterior doors with steel doors comparable to what is there.  This is to include new hinges and hardware.  Paint doors inside and out.

- Paint exterior of the building to include the flashing and coping along the roof line, the gutters, (down spouts and scuppers) and the supports for the awnings (door entry canopy's) at all entrances

- Repair and replace glazing compound in exterior windows (exterior glazing compound probably has asbestos. Remove and re-caulk all windows and around all wall vents.)

- Repair awnings (door entry canopy's) over exterior doorways except the long one at the front entrance. Replace the soffits with pressure treated 3/8" AC plywood and all associated wood trim and paint. Repair and re-flash the roof to wall flashing.

- Replace wood siding, caulk and paint at the top of exterior windows on all windows at which the siding exists.

- Repair roof as noted in the roof inspection report. There is at least one place that may be leaking and needs sealing. A copy of the roof inspection report can be obtained from the Windsor Town Hall or by contacting Michael Stallings at 757-242-4288.

- Remove all door number signs point up any holes from sign fasteners.

Required Specifications

# Paint

Benjamin Moore “Aura” paint for brick in two (2) colors to be matched to two colors on the new school:  (1)  the beige color on the new school will be used for all trim to include doors, the flashing board along the roofline, the gutters, and the vertical brick support buttress sections at intervals along the brick exterior walls;  and (2)  the “burgundy” color of the bricks on the new school will be used to paint the rest of the brick on the old gymnasium.

# Submission Information

- Proposals must contain an itemized list of work to be completed with prices for each task listed above.

- Proposals can be submitted in person at Windsor Town Hall or mailed to:

Michael Stallings

P. O. Box 307

Windsor, VA 23487

- Interested firms may contact Michael Stallings at 757-242-4288 with questions or to schedule a visit to the site.

- Proposals will be accepted until August 22nd.